Intelligent Robotics: Assignment 1

Riccardo Simion1, Andrea Scanu1, Riccardo Zerbinati1

1Università degli Studi di Padova

E-mail: riccardo.simion@studenti.unipd.it , andrea.scanu.1@studenti.unipd.it, riccardo.zerbinati@studenti.unipd.it

Introduction

This work presents a complete perception–navigation pipeline for a TurtleBot3 robot operating in the IASLab simulated environment. At initialization, the robot spawns at the entrance of the lab with the objective of autonomously localizing two AprilTags, navigating to a stable and collision-free pose centered between them, and subsequently detecting three cylindrical tables distributed in the room. The system is required to return the positions of all tables expressed in the odom frame.

Three challenges shape the system design. First, AprilTag detections must be converted into accurate 3D positions using camera intrinsics. Second, the navigation pipeline must reliably reach a narrow target region between two physical markers without colliding with them. Third, table detection must be robust to partial scans and sensor noise, motivating the development of a geometry-based method rather than relying solely on centroid calculation. In response to these challenges, we designed a three-stage event-driven architecture in which each stage publishes a completion signal that triggers the next.

# System Architecture

The information flow is strictly unidirectional, with each module consuming and producing specific events that mediate inter-node coordination.

The AprilTag detection system continuously receives AprilTag detections from the service camera, processes them into 3D poses, and publishes a navigation target on the /target topic. This triggers the driver node, which interfaces with the Nav2 stack through the navigate_to_pose action server. Upon reaching the computed goal, the driver node emits an /arrived signal that activates the table detection module. The final positions of the detected cylindrical tables are published as a list of poses in the odom frame on the /detected_tables topic.

# Components Description

2.1: apriltag_manager

Apriltag_manager_node aim is to provide to the navigation system the appropriate position relative to the apriltags positioned in the simulation and relative to the map frame.

To do so first the node subscribes to the /rgb_camera/camera_info and apriltag/detections topics, then fetches the rgb camera intrinsics from the first topic and stops to read the CameraInfo messages when these data are saved, since we expect the camera to be always the same while the simulation is running.

When the camera intrinsics are available the node will start to process the AprilTagDetections messages:

for every message received first it uses the cv::solvePnP() function to obtain from the detections data and the camera intrinsics data the positions of the apriltags relative to the rgb camera, then it computes the mean position of these and convert this position to the map frame, finally it applies to the obtained position an offset on the x and y coordinates in order to not send the robot excactly between the two apriltags and sets the z coordinate equal to the odom z coordinate, since the robot can’t fly. Finally, the obtained position is published on the /target topic ready to be used by the driver_node.

# 2.2: driver_node:

driver_node is responsible for bringing the robot to the designated target pose in the map, handling Nav2 startup, goal forwarding, and arrival reporting. At first we encountred a lot of problem with the nav2 initialization and the goal sending. After setting up the logs for understand better what happens, heres the final version of the pipeline:

- Start Nav2: Calls ManageLifecycleNodes on the localization and navigation managers with STARTUP and waits for all nodes to become active.

- Set Initial Pose: Publishes /initialpose with default position/yaw once Nav2 is ready.

- Receive Target: Subscribes to target (geometry_msgs/PoseStamped) in the map frame.

- Send Goal: Forwards the target to /navigate_to_pose via the Nav2 action client.

- Track Navigation: Logs feedback (distance remaining) and, on success, publishes arrived = true.

# 2.3: table_detector:

table_detector purpose is to detect cylindrical tables using LiDAR data and output their centers in the odom frame. Our first approach was a straightforward approach that computed the centroid of laser returns associated with each table, but this method suffers from systematic geometric bias: the centroid of an arc does not necessarily coincide with the center of the corresponding circle.

To address this limitation, we employ a Least Squares circle fitting method (Kasa’s algorithm [1]), which minimizes

i=1N[(xi-cx)2+(yi-cy)2-r2]2

This approach yields unbiased estimates of both center and radius, even when only partial arcs are visible. The detection pipeline proceeds as follows:

- Clustering: Laser points are grouped based on local continuity, with a threshold of 0.30 m ensuring separation from walls and background.

- Filtering: Clusters with insufficient points or overly large extents are discarded.

- Circle Fitting: A Kasa fit is applied to each cluster, extracting candidate centers and radii.

- Validation: A radius constraint of 0.10–0.60 m filters out non-table structures and walls (the latter often yield near-infinite radii).

- Frame Transformation: TF2 converts cluster centers from base_scan to odom.

- Output: The results are published as a PoseArray containing up to three table positions.

Empirically, the fitted centers remain stable across multiple trials and robot orientations.

# Justification for Implementation

The proposed pipeline is the result of several design decisions driven by reliability, modularity, and clarity of data flow.

We adopted a strictly modular, event-driven architecture to guarantee that each stage of the pipeline operates only when the previous one has successfully completed. This prevents race conditions and eliminates hidden dependencies between components. By using lightweight event topics such as /target and /arrived, each module is isolated from the internal logic of the others, simplifying debugging and allowing the system to be extended without modifying other nodes.

For example, table detection is activated only once navigation is complete. This design reduces unnecessary computation, avoids interference with navigation, and guarantees that all table detections are performed from the final, stable pose requested by the assignment.

# Results and conclusions

The system reliably detects the two AprilTags at the entrance and computes stable 3D poses using camera and TF transformations. Across all trials, AprilTag-based goal estimation remained accurate, consistently placing the target near the tags.

Navigation through Nav2 performed robustly. Proper lifecycle initialization and initial pose setup were essential to ensuring predictable behaviour.

The table detector proved resilient to noisy or partial LiDAR scans. The algorithm consistently identified all three cylindrical tables, with stable results across repeated runs.

Overall, the pipeline operated reliably end-to-end. The event-driven modular architecture ensured clean coordination between perception, navigation, and detection.

- Acknowledgments

Parts of the text in this report were revised with the assistance of ChatGPT (OpenAI) to improve clarity, readability, and overall linguistic quality. The AI was used exclusively as a writing aid and did not influence the technical content, analyses, results, or design decisions described in this work.

# References

[1]  E. Kasa, A curve fitting procedure and its error analysis, IEEE Transactions on Instrumentation and Measurement, 1976.

# Links

- GitHub repository:

https://github.com/Zerby0/2_assignment_1.git

- Link to the video:

2_assignment_1/two_assignment_1/video_simulation.mp4 at master · Zerby0/2_assignment_1