Intelligent Robotics: Assignment 1

Riccardo Simion1, Andrea Scanu1, Riccardo Zerbinati1

1Università degli Studi di Padova

E-mail: riccardo.simion@studenti.unipd.it , andrea.scanu.1@studenti.unipd.it, riccardo.zerbinati@studenti.unipd.it

Introduction

This work presents a complete perception–navigation pipeline for a TurtleBot3 robot operating in the IASLab simulated environment. At initialization, the robot spawns at the entrance of the lab with the objective of autonomously localizing two AprilTags, navigating to a stable and collision-free pose centered between them, and subsequently detecting three cylindrical tables distributed in the room. The system is required to return the positions of all tables expressed in the odom frame.

Three challenges shape the system design. First, AprilTag detections must be converted into accurate 3D positions using camera intrinsics. Second, the navigation pipeline must reliably reach a narrow target region between two physical markers without colliding with them. Third, table detection must be robust to partial scans and sensor noise, motivating the development of a geometry-based method rather than relying solely on centroid calculation. In response to these challenges, we designed a three-stage event-driven architecture in which each stage publishes a completion signal that triggers the next.

# System Architecture

The information flow is strictly unidirectional, with each module consuming and producing specific events that mediate inter-node coordination.

The AprilTag detection system continuously receives AprilTag detections from the service camera, processes them into 3D poses, and publishes a navigation target on the /target topic. This triggers the driver node, which interfaces with the Nav2 stack through the navigate_to_pose action server. Upon reaching the computed goal, the driver node emits an /arrived signal that activates the table detection module. The final positions of the detected cylindrical tables are published as a list of poses in the odom frame on the /detected_tables topic.

ADD UML

Figure 1:

# Components Description

2.1: apriltag_manager

# 2.2: driver_node:

driver_node creates the necessary publishers and subscribers and initializes the action client for Nav2’s navigate_to_pose action.

Then, it publishes the robot’s start position on the /initialpose topic. This gives Nav2 a reference point for localization.

Once initialized, the node listens on the /target topic. The message received represents the location the robot should navigate to.

Upon receiving a target pose, the node first checks whether it is already executing a navigation task. If so, it ignores the new request.

If it’s free, it waits for the Nav2 action server to become available, computes the initial straight-line distance between the initial position and the goal and then sends the navigation goal to Nav2.

At this point, it sets a flag indicating that the robot is now moving.

While the robot is moving it receives continuous updates about the remaining distance. Using this, the node computes the navigation progress percentage and logs it. When it arrives at the goal it publishes to /arrived a true. If navigation fails or gets canceled, it logs a warning. In both cases, it clears the internal state and resets the navigation flag. After the navigation concludes, the node is ready again and can accept the next target without issues.

# 2.3: table_detector:

table_detector purpose is to detect cylindrical tables using LiDAR data and output their centers in the odom frame. Our first approach was a straightforward approach that computed the centroid of laser returns associated with each table, but this method suffers from systematic geometric bias: the centroid of an arc does not necessarily coincide with the center of the corresponding circle.

To address this limitation, we employ a Least Squares circle fitting method (Kasa’s algorithm), which minimizes

i=1N[(xi-cx)2+(yi-cy)2-r2]2

This approach yields unbiased estimates of both center and radius, even when only partial arcs are visible. The detection pipeline proceeds as follows:

- Clustering: Laser points are grouped based on local continuity, with a threshold of 0.30 m ensuring separation from walls and background.

- Filtering: Clusters with insufficient points or overly large extents are discarded.

- Circle Fitting: A Kasa fit is applied to each cluster, extracting candidate centers and radii.

- Validation: A radius constraint of 0.15–0.45 m filters out non-table structures and walls (the latter often yield near-infinite radii).

- Frame Transformation: TF2 converts cluster centers from base_scan to odom.

- Output: The results are published as a PoseArray containing up to three table positions.

This method proves resilient to noise, partial occlusion, and irregular scanning geometry. Empirically, the fitted centers remain stable across multiple trials and robot orientations.

# Justification for Implementation

TO DO: WRITE REASONS WHY WE CHOOSED THIS IMPLEMANTATION; STRENGHTS ECC…

# Results and conclusions

TO DO: WRITE RESULTS AND CONCLUSIONS

# References

[1]